Design Documentation

[GitHub]: https://github.com/DanielJ0131/interactive-house/tree/main/SG3_Devices/SG3_Artifacts

Subgroup 3: Devices

Revision History	1

Design item List	2

Design Item Descriptions	2

D1. High-Level Architecture (Device → Gateway → Server → UI)	2

D2. Arduino Device Component Design	4

D3. Serial Communication Protocol	4

D4. Interaction Flow – UI Command to Device	6

D5 Fault Handling & Safe Behaviour	8

Subgroup 3: Devices

Revision History

| Date | Version | Description | Author |
| --- | --- | --- | --- |
| 2026-02-19 | 1.0 | Initial design document | Patrick Nordahl |
| 2026-03-10 | 1.1 | Minor terminology refinement for clarity and consistency | Patrick Nordahl |
| 2026-04-14 | 1.2 | Add refactored Gateway architecture | Patrick Nordahl |
| 2026-05-08 | 1.3 | Refactored Arduino architecture with Observer Pattern; updated Gateway to Node.js. | Mustafa Al-Bayati, Ryad Hazin, Tony Nguyen, Sergej Macut |
| 2026-05-08 | 1.4 | Updated file to contain Table of content. | Mustafa Al-Bayati, Ryad Hazin, Tony Nguyen, Sergej Macut |

Design item List

| Requirement Name | Priority |
| --- | --- |
| D1. High-Level Architecture (Device → Gateway → Server → UI) | Essential |
| D2. Arduino Device Component Design | Essential |
| D3. Serial Communication Protocol (Message types + structured format) | Essential |
| D4. Interaction Flow UI command to Devices | Essential |
| D5. Fault Handling & Safe State Behavior (Communication failure strategy) | Essential |

Design Item Descriptions

D1. High-Level Architecture (Device → Gateway → Server → UI)

Description: The system follows a layered architecture designed for modularity and asynchronous data flow. Communication is routed through a dedicated Gateway, which has been refactored from a Python-based script to a Node.js (JavaScript) environment. This change leverages Node’s non-blocking I/O to handle simultaneous Serial and WebSocket communication more efficiently.

The architecture consists of four logical layers:

- Device Layer (Arduino): Handles real-time sensor acquisition and actuator control. It utilizes a modular C++ structure with dedicated .h and .cpp files for each component.

- Gateway Layer (Node.js): Acts as the middleman. It parses structured Serial messages (OBS/ACK), validates the data, and forwards it to the Server Layer via HTTP/WebSockets.

- Server Layer: Centralizes system state and manages command distribution to clients.

- User Interface Layer: Provides the dashboard for real-time monitoring and manual overrides.

Figure 1. Architecture/Component Diagram (D1)

This design ensures separation of concerns and supports interoperability through the middleman architecture.

D2. Arduino Device Component Design

Description: The Arduino firmware has been refactored into a modular, object-oriented structure to ensure scalability and maintainability. Each hardware component (e.g., DoorDevice, GasSafety, LCDManager) is encapsulated in its own class across separate header (.h) and source (.cpp) files.

Key Architecture: Observer Design Pattern To reduce coupling between sensors and actuators, the system implements the Observer Design Pattern:

- Subjects (Sensors/Logic): Components like GasSafety and SteamSensor act as Subjects. They monitor thresholds and "notify" a list of subscribers when an event occurs.

- Observers (Actuators): Devices like Fan, Buzzer, Door, and Window implement an Observer interface. They wait for notifications and react independently when triggered.

Modular Components:

- Core Logic (Observer.h, Subject.h): Provides the base classes for the event-driven system.

- Sensors & Safety: Encapsulates the logic for threshold checking and state notification.

- Devices: Individual classes for actuators that handle physical hardware changes (PWM, digital writes).

- LCDManager: A dedicated display controller that aggregates "Active States" from the system to provide real-time user feedback.

D3. Serial Communication Protocol

Description:

Structured Serial communication is used between the Arduino device and the gateway to ensure reliable data exchange.

Three conceptual message types are defined:

- OBS (Observation) – Sent from device to gateway (sensor values + state).

- CMD (Command) – Sent from gateway to device (state change request).

- ACK (Acknowledgement) – Sent from device to confirm applied command.

Example key-value format:

- obs;gas=312;motion=1;light=0

- cmd;light=1

- ack;light=1

Final message format (JSON or key-value) remains an open design decision.

Structured messaging reduces integration risks and supports validation.

Figure 2. Class diagram (D2+D3)

Figure 3. Refactored Gateway Architecture.

D4. Interaction Flow – UI Command to Device

Description: The interaction flow describes how the system synchronizes state between the high-level UI and the low-level hardware using an event-driven approach.

Scenario 1: Automated Event (Sensor Trigger)

- Detection: SteamSensor detects moisture (Rain).

- Notification: The sensor (Subject) calls its notify() method.

- Update: All attached Observers (DoorDevice, WindowDevice) receive the update and trigger their "Close" logic.

- Feedback: The Arduino sends an OBS (Observation) or ACK message through the Node.js Gateway to update the UI state.

Scenario 2: Manual Command (UI Toggle)

- UI: User clicks "Toggle Light."

- Gateway: The Node.js gateway receives the command from the server and writes a CMD;light=1 string to the Serial port.

- Arduino: The CommandHandler decodes the message and instructs the ActuatorManager to apply the state change.

- Verification: The system sends an ACK back to ensure the UI reflects the actual hardware state.

Figure 3. Sequence Diagram (D4)

Figure 4. State Diagram (Arduino Device)

This flow ensures end-to-end synchronization between UI, server and device.

D5 Fault Handling & Safe Behaviour

Description:

To improve reliability, the device must handle communication failures gracefully.

If Serial communication is interrupted or corrupted:

- The Arduino continues operating in its last known safe state.

- Incoming commands must be validated before actuation.

- Invalid messages are ignored.

- The gateway is responsible for dropping corrupted data and logging errors.

This design reduces the impact of communication instability and addresses identified integration and reliability risks.